10 RULES OF ROCKET STOVE COMBUSTION

By Dr. Larry Winiarski

- Don’t heat more wood than you are burning. Don’t heat too much wood (surface area).

- Get the right amount of air. Not too much, not too little.

- Air should enter with velocity. Like blowıng on the coals.

- Air/fuel/air/fuel…..mixing.

- Air interleaved or underneath the wood.

- Wood arranged, 2-3 pieces of wood with spaces for air. People in bed keeping each other warm.

- All combustion to take place in insulated low mass surroundings. Don’t lose heat. Don’t absorb heat at startup.

- Complete combustion at high temperature, then extract heat.

- Vertical chimney, insulated and hot provides strong draft, more time for combustion. This is the big secret of the Rocket Stove.

- Air flow passages the same cross section.

- Feed opening sized for power output of stove. Too large= too much cold air, too little velocity. You need the velocity to keep the flames going up and mixıng air.

- Dry wood. 2-3 stıcks for radiatıon. After starting, not too much surface area.